PRODUCT REQUIREMENTS DOCUMENT

ERC Rover Control GUI

Desktop Application — Dear ImGui / C++ / ROS 2

| Document Version | 1.2 |
| --- | --- |
| Date | April 2026 |
| Audience | Development Team & ERC Competition Judges |
| Competition | European Rover Challenge (ERC) |
| Technology Stack | C++ with Dear ImGui, ROS 2 (communication) |
| Status | Draft |

Purpose This document defines the functional and non-functional requirements for a GUI application used to operate a competition rover across four distinct task modes: Science, Navigation, Maintenance, and Probing. Ergonomics, usability, and operator efficiency are primary design goals in line with ERC evaluation criteria.

# 1. Project Overview

The ERC Rover Control GUI is a desktop application built in C++ using Dear ImGui. All communication with the rover is handled via ROS 2 topics and services. The application is structured around four operational views, each tailored to a specific ERC competition task, minimising cognitive load and maximising operator efficiency under competition pressure.

## 1.1 Goals & Success Criteria

- Ergonomics: Intuitive interface operable under competition pressure with minimal training.

- Task Clarity: Each task has a dedicated view showing only relevant controls and data.

- Reliability: All critical controls accessible within 1–2 interactions at all times.

- Competition Score: GUI positively contributes to ERC ergonomics and usability scoring criteria.

## 1.2 Technology Stack

| Language | C++17 or later |
| --- | --- |
| GUI Framework | Dear ImGui (immediate-mode GUI) |
| Communication | ROS 2 — topics for telemetry/sensor streams, services for commands |
| Input | USB Gamepad (primary, all views) + keyboard shortcuts (fallback) |
| Camera / Video | OpenCV via ROS 2 image topics |
| Build System | CMake with ROS 2 ament_cmake |

# 2. Persistent UI Elements

The following elements are always visible in a fixed status bar at the top of the window, regardless of the active view. They are never hidden or obscured by view-specific content.

| Feature | Priority | Notes |
| --- | --- | --- |
| Emergency Stop (E-STOP) | Must Have | Large high-contrast button always on screen. Halts both drive and arm. Dedicated gamepad button. Red zone, never obscured. |
| Battery Level | Should Have | Percentage + icon, colour-coded: green >50%, yellow 20–50%, red <20%. Flashes when critical. |
| Signal / Connection Status | Must Have | ROS 2 link state + latency (ms). All controls disabled with warning banner if connection is lost. |
| Current Speed | Must Have | Live rover speed readout (m/s). Visible at all times; enlarged in Probing view. |
| Active View Label | Must Have | Clearly shows which task view is active to prevent confusion under pressure. |
| Drive / Arm Mode | Must Have | Shows current gamepad mode: DRIVE or ARM. Applies to Science, Maintenance, and Probing views. |
| Gamepad Connected | Should Have | Icon indicating gamepad detection state; keyboard fallback activates if disconnected. |

Layout note The persistent bar is approx. 44 px tall. E-STOP occupies a visually distinct red zone at the far right. The four view tabs are part of this bar. Drive/Arm mode indicator uses a large colour badge (blue = Drive, orange = Arm) so it is legible at a glance.

# 3. Application Views — Overview

Four views are selectable from the persistent top bar. Switching is instant. Switching during an active recording prompts confirmation to prevent data loss.

| View | Camera | Drive | Arm | Coordinates | Key Feature |
| --- | --- | --- | --- | --- | --- |
| Science | Yes | Gamepad | Secondary | Live GPS | Sampling, weight, color inspection |
| Navigation | No | Gamepad | No | Estimated (odometry) | Autonomous mode + path map |
| Maintenance | Yes | Gamepad | Primary | Live GPS | Arm teleoperation of external panel |
| Probing | Yes | Gamepad | Secondary | No | Sprint mode + probe pickup |

Shared arm control pattern (Science, Maintenance, Probing) A single gamepad button toggles between DRIVE mode and ARM mode. In ARM mode: sticks control arm joints or end-effector, triggers control gripper. The Drive/Arm mode indicator in the persistent bar makes the current mode unambiguous. In Maintenance, ARM mode is the default on view entry; in Science and Probing, DRIVE mode is the default.

# 4. View: Science

The Science task requires the rover to identify soil samples by color, collect deep core samples via drill, collect water samples, place samples into onboard containers, and measure collected sample weight. Visual accuracy and situational awareness are critical — color rendering and image clarity are must-have requirements.

## 4.1 Operational Context

Task description The operator visually inspects terrain via the camera feed to identify samples of the required color. They then position the rover and use the robotic arm to drill a core sample or collect a water sample. The sample is placed into an onboard container. The GUI must display container occupancy and the weight of the collected sample(s) as confirmed by the onboard scale.

## 4.2 Feature Requirements

| Feature | Priority | Notes |
| --- | --- | --- |
| Live Camera Feed | Must Have | Full-resolution color-accurate stream. Color fidelity is critical for sample identification by sand/soil color. |
| White Balance / Color Cal. | Should Have | Manual white balance control to ensure true color representation under varying outdoor lighting conditions. |
| Brightness & Contrast Adj. | Should Have | Real-time adjustments to reveal water surface details and dark terrain features. |
| Digital Zoom | Should Have | Software zoom for close inspection of sample color or water surface before approach. |
| Video Recording | Should Have | Toggle record; red dot + elapsed time. Written incrementally to disk. Records science observations for review. |
| Photo Capture | Must Have | Single-button snapshot with auto-timestamp for documenting sample sites. |
| Snapshot Thumbnail Strip | Should Have | Recent captures visible below feed for quick reference without leaving the view. |
| Live GPS Coordinates | Should Have | Real-time GPS position (lat/lon or local X/Y/Z). Prominently displayed. Marked stale if ROS 2 topic silent >2 s. |
| Rover Drive Control | Must Have | Full gamepad drive control for positioning rover at sample sites. Default mode on view entry. |
| Robotic Arm Control | Must Have | Gamepad ARM mode: joint or end-effector control for drill positioning, sample pickup, and container placement. |
| Arm Joint Angles | Should Have | Live readout of all joint angles. Visual arm state diagram. |
| Gripper State & Force | Should Have | Open/close state and grip force — critical for delicate sample handling. |
| End-Effector Position | Must Have | XYZ position of arm tip for precise drill placement and container loading. |
| Joint Limit Warnings | Should Have | Colour highlight when joint nears limit. |
| Drive / Arm Mode Indicator | Must Have | Persistent badge showing current gamepad mode: DRIVE or ARM. |
| Drill State Display | Should Have | Current drill status: Idle / Drilling / Complete / Error. Drill depth readout if sensor available. |
| Drill Control | Must Have | Start / stop drill command. Accessible in ARM mode without switching views. |
| Container Occupancy | Must Have | Visual display of onboard sample container slots: empty / occupied. Shows which slots are filled. |
| Sample Weight Readout | Must Have | Live weight from onboard scale in grams. Updated after each sample placement. |
| Water Sample Mode Indicator | Should Have | Visual cue when operator switches to water sampling procedure (different arm motion profile). |
| Arm Preset Positions | Should Have | Named poses: Stow, Drill Approach, Container Load. One-button activation. |

## 4.3 Layout Notes

- Camera feed: 55–65% of screen width. Color accuracy is more important than large size.

- Arm telemetry panel (joints, gripper, end-effector) in a collapsible right-side panel — expanded by default in ARM mode.

- Sample data panel (drill state, container occupancy, weight) docked below camera feed.

- Record and Photo buttons: large, one-click.

- GPS coordinates: fixed top-left or alongside persistent bar — always visible.

- White balance and brightness controls: accessible as a compact overlay, not buried in a menu.

# 5. View: Navigation

The Navigation view is used when the rover operates autonomously. No camera feed is shown. The operator monitors estimated position, observes the planned path, and retains gamepad override capability at all times. No arm control is available in this view — the arm should be in stow position during transit.

## 5.1 Feature Requirements

| Feature | Priority | Notes |
| --- | --- | --- |
| Rover Control (Gamepad) | Must Have | Manual gamepad override always active, even during autonomous operation. |
| Estimated Coordinates | Must Have | Continuously updated X, Y, heading from ROS 2. Clearly labelled 'Estimated'. Large display — primary data here. |
| Autonomy Status Banner | Must Have | Full-width colour banner: Autonomous (blue) / Manual Override (orange) / Idle (grey). |
| Path / Waypoint Map | Should Have | 2D top-down map showing planned route, waypoints, and current estimated position via ROS 2 nav_msgs/Path. |
| Speed & Heading Readout | Must Have | Current speed and heading; enlarged vs persistent bar since no camera competes for space. |
| Waypoint Controls | Should Have | Add, remove, or reorder waypoints. Publishes updated path to ROS 2. |
| Emergency Stop (E-STOP) | Must Have | Via persistent bar — always available. |

## 5.2 Layout Notes

- Freed camera space used for large map/path visualization or oversized telemetry.

- Coordinates displayed in the largest font size of any view — primary situational tool.

- Autonomy status banner spans full width below the persistent bar.

# 6. View: Maintenance

The Maintenance task requires the operator to use the rover's robotic arm to interact with an external robot's interface panel — pressing buttons, turning dials, connecting cables, or reading indicator states. Arm control is the primary activity. The rover may need repositioning, so drive control remains available via gamepad mode toggle.

## 6.1 Operational Context

Task description The operator docks the rover near a separate robot's control panel. Using ARM mode on the gamepad, they teleoperate the robotic arm to manipulate the external panel's controls. The camera provides visual guidance for precise arm placement. Rover drive control is available via mode toggle for repositioning. ARM mode is the default on view entry.

## 6.2 Feature Requirements

| Feature | Priority | Notes |
| --- | --- | --- |
| Live Camera Feed | Must Have | Feed aimed at external panel and arm end-effector. |
| Drive Control (Gamepad) | Must Have | Drive mode accessible via gamepad. |
| Robotic Arm Control | Must Have | Primary control in this view. Joint or Cartesian end-effector control. |
| Arm Joint Angles | Should Have | Live per-joint angle readout. Visual arm silhouette or bar indicators. |
| Gripper State & Force | Should Have | Open/close state and grip force — prevents over-gripping panel components. |
| End-Effector Position (XYZ) | Must Have | Cartesian position of arm tip for precise panel interaction. |
| Joint Limit Warnings | Should Have | Colour gradient per joint approaching limit (green > yellow > red). |
| Drive / Arm Mode Indicator | Must Have | Dominant colour badge: ARM (orange, default) / DRIVE (blue). |
| Motor Temperature & Current | Should Have | Per-joint motor temperature and current draw. Sustained arm use risks overheating. |
| Coordinate Display | Must Have | Rover position for docking/approach to the external panel. |
| Arm Preset Positions | Should Have | Named poses: Stow, Approach Panel, Work Position. One-button activation. |
| External Panel Camera Zoom | Should Have | Digital zoom on camera feed for reading indicator states on target panel. |

## 6.3 Layout Notes

- Camera feed left; arm telemetry panel right — both always visible simultaneously.

- Drive/Arm mode indicator: colour change across a large top-of-view area, impossible to miss.

- Joint limit proximity: colour gradient bars, not just numbers.

- Arm preset buttons: large, clearly labelled, accessible without leaving ARM mode.

# 7. View: Probing

The Probing task requires the rover to locate physical probe objects on the ground and collect them using the robotic arm. Speed of movement between probe locations is critical. The camera must provide a clear, wide view for spotting probes on the ground. Arm control handles the pickup.

## 7.1 Operational Context

Task description The rover navigates a field to find physical probes (small objects) placed on the ground. The operator uses the camera feed to spot a probe, drives quickly to its location (Sprint mode), switches to ARM mode to pick it up with the gripper, then sprints to the next probe location. Speed and visual coverage are the dominant concerns.

## 7.2 Feature Requirements

| Feature | Priority | Notes |
| --- | --- | --- |
| Live Camera Feed | Must Have | Large, wide field-of-view feed for spotting probes on the ground. Full-screen toggle available. |
| Rover Control (Gamepad) | Must Have | Full drive control. Sprint mode accessible without mode switch. |
| Sprint / Boost Mode | Should Have | Dedicated gamepad button for maximum drive speed. Visually indicated with orange badge + enlarged speed readout. Active until released. |
| Speed Presets | Should Have | Two switchable profiles: Precision (slow, fine) and Sprint (max speed). Active profile always displayed. |
| Robotic Arm Control | Must Have | Gamepad ARM mode for probe pickup: end-effector positioning and gripper control. |
| Gripper State & Force | Should Have | Confirms probe is gripped. Prevents dropping during transit. |
| End-Effector Position | Must Have | XYZ position for reliable probe pickup alignment. |
| Arm Joint Angles | Should Have | Joint readout for arm state awareness during pickup. |
| Joint Limit Warnings | Should Have | Colour warning when joint nears limit during pickup manoeuvre. |
| Drive / Arm Mode Indicator | Must Have | Clear mode badge — critical since operator switches rapidly between driving and picking up. |
| Probe Collection Counter | Should Have | Tally of probes collected in the current run. Simple numeric display. |
| Full-Screen Camera Toggle | Should Have | Maximises camera feed for terrain scanning; HUD overlaid transparently. |
| Arm Preset Positions | Should Have | Named poses: Stow (for driving), Pickup Approach. One-button activation. |

## 7.3 Layout Notes

- Camera feed as large as possible — probe spotting is visually demanding.

- Sprint mode activation unmistakeable: full-speed badge, colour change on speed readout.

- Arm telemetry compact by default (gripper + end-effector only); expands to full panel in ARM mode.

- Probe counter visible at all times — small but persistent.

- Persistent status bar (E-STOP, battery, signal, speed) always at top.

# 8. UX & Ergonomics Requirements

Ergonomics is an explicit ERC scoring criterion. The requirements below address layout consistency, visual design, gamepad-first interaction, and safe operation under competition conditions.

## 8.1 Navigation & Layout Consistency

- All four views accessible within one button press from any view.

- Persistent bar (E-STOP, battery, signal, speed, mode) always visible — never hidden.

- Rover control and arm telemetry widgets occupy consistent screen positions across all views to build muscle memory.

- No critical control requires scrolling or menu navigation.

## 8.2 Visual Design

- Dark theme by default — reduces eye strain in bright outdoor competition environments.

- Consistent colour semantics across all views: red = danger/stop, orange = ARM mode/sprint active/warning, yellow = caution, green = nominal, blue = DRIVE mode/informational.

- Minimum font sizes: 14 px for data labels, 18 px for critical values (speed, coordinates, mode badges).

- Camera feed renders at 30+ FPS without tearing or visible latency.

- Stale ROS 2 data (no update >2 s): value shown in grey or italicised. Never rely on stale values silently.

## 8.3 Gamepad & Input

- Primary input: USB gamepad for all rover drive and arm control in every view.

- Mode toggle: Single gamepad button switches between DRIVE and ARM mode. Prominent visual confirmation on every mode change.

- Immediate feedback: Every gamepad or keyboard action produces visible on-screen change within one frame.

## 8.4 Error & Edge Case Handling

- Camera lost: 'No Signal' placeholder immediately — never a silent frozen frame.

- ROS 2 connection lost: all controls disabled, full-width warning banner, automatic reconnection with retry counter.

- Gamepad disconnected: immediate warning; keyboard fallback activates automatically.

- Critical battery (<20%): persistent bar flashes red.

- All recorded video written incrementally — a crash must not corrupt or lose an entire recording.

- Application must not crash during a competition run — wrap all ROS 2 subscriptions and camera I/O in error recovery.

# 9. Non-Functional Requirements

## 9.1 Performance

| Feature | Priority | Notes |
| --- | --- | --- |
| UI Frame Rate | Must Have | Minimum 30 FPS; target 60 FPS. ROS 2 callbacks must not block the ImGui render thread. |
| Camera Latency | Must Have | End-to-end display latency < 500 ms under normal network conditions. |
| Control Latency | Must Have | Drive and arm commands published to ROS 2 within 50 ms of gamepad input. |
| Sensor Update Rate | Must Have | GPS, joint states, and weight sensor displayed at their native ROS 2 publish rate (no artificial downsampling). |
| Startup Time | Should Have | Application fully ready (ROS 2 nodes connected) within 10 seconds of launch. |

## 9.2 Reliability

- Handle loss of any ROS 2 topic (camera, GPS, arm) gracefully — show stale/no-signal state without crashing.

- Video recording written incrementally; crash must not lose entire session.

- ROS 2 node reconnection automatic with visible retry status.

## 9.3 Portability

- Primary target: Linux Ubuntu 22.04+ with ROS 2 Humble or later.

- Build via CMake with ament_cmake for ROS 2 integration.

# 11. Open Questions

| OQ-01 | What are the exact ROS 2 topic names and message types used by the rover? A full topic list is needed to implement subscriptions and publishers. |
| --- | --- |
| OQ-02 | How many cameras does the rover have? Are multiple feeds available simultaneously (e.g. forward + arm-mounted)? |
| OQ-03 | Does the robotic arm support Cartesian end-effector control via ROS 2, or only joint-by-joint commands? |
| OQ-04 | What coordinate system does the GPS/odometry provide? (lat/lon/alt vs. local X/Y/Z frame) |
| OQ-05 | How many onboard sample container slots are there? What states can each slot report (empty / occupied / type)? |
| OQ-06 | What is the maximum drive speed of the rover? Required to calibrate Sprint mode in Probing. |
| OQ-07 | What file format should be used in the Science task? |
| OQ-08 | What ROS 2 topic provides drill depth and drill state? Is water sampling a different hardware mode? |

End of Document — ERC Rover Control GUI PRD v1.2